- Scope

This document describes the Oplink Switch Equipment Command Set for use in communicating with and controlling intelligent optical switches.

- Abbreviations

The following abbreviations are used in this standard:

ASCII:  American standard character for information interchange

- Communication Interface

The RS232 serial interface shall support the following settings as default:

| Data rate (baud) | 115200 |
| --- | --- |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |
| Flow control | None |

  - Command Syntax

    - Command Set Encoding Language

All data sent to/from the module are encoded in 8-bit ASCII characters. Refer to ISO/IEC 8859-1.

    - Command set syntax structure

Commands consist of a command string, zero or more arguments, and end with a terminator.
Command strings are case-insensitive. All commands have the following format: 
COMMAND <argument_1> <argument_2> <argument_3> ………<CR><LF>.

    - Command Arguments

Arguments may be strings of letters, integers or floating-point numbers. Floating-point

numbers shall be entered in fixed notation (e.g. 0.023 not 2.3e-2).

Command strings and arguments shall be separated by one separator character. Separators shall be 8-bit ASCII space characters.

    - Command Set Terminators

The command terminator is <CR><LF>. In the case of <CR><LF>, the command is executed on receipt of the <CR> character and the <LF> character is ignored.

A successful command execution may be responded to with an “OK<CR><LF>”

Unacknowledged commands may be responded to with a “Error message <CR><LF> as below”.

- "OK"

- "Err: Invalid Command"

- "Err: Invalid Parameter"

- "Err: Invalid Data"

- "Err: Busy"

- "Err: Execute Fail"

- "Err: Not Online"

Command response consists of an optional response or error message.

    - Module response prompt

All responses are followed by a prompt, which indicates that the module is ready to process another command. The default prompt is <CR><LF> “>”.

    - Echo mode

By default, the module doesn’t echo characters sent to it. However, if echo mode is enabled, printable 8-bit ASCII characters are echoed. Also, echo mode enables command-line editing using the backspace key (the backspaces are echoed if there are characters to delete).

    - Remote Mode

As soon as the module receives the data from the RS232 bus, it will enter Remove mode. This mode can be removed by press both “” and “” keys at the same time.

    - Setting Restoration

After power on and soft reset, all latching switches keeps previous states. For non-latching switches, it will set back to its default state.

    - Alarm

If the switch setting position can’t reach, the buzzer will be on. The buzzer can be stopped by press both “” and “” keys at the same time.

    - Command Set

This clause describes the syntax of commands. The commands are informative. Each command is shown with arguments in italics. All Switches shall support commands shown in this clause. Variations due to optional arguments are shown on separate lines. In the examples, a sample of communications is shown starting with the prompt from the module and ending with the prompt following the final command. Text sent to the module from the host is shown in bold (assumes echo mode is on).

Table 1 shows the software commands.

| IDX | Command | Description | Example |
| --- | --- | --- | --- |
| 1 | INFO INFO X (Optional) | Get module information. Response are including below items. SN: PN: DRV: (Driver of this module) OPT: (Optical of this module) FW Ver: HW Ver: Mfg Date: Cal Date: Every item is 32 bytes, 0~31 Bytes: Description 31~63 Bytes: Items left alignment, pad right with 0x00. Set manufacture to X. X is fixed to 256 bytes. CRC32 value will be calculated and verified internally. (Valid after passwd command) | >INFO Response: SN: Z1234567 PN: Switch DRV: Defined by GUI OPT: Defined by GUI FW: 1.0.0.0 HW Ver: 1.0 Mfg Date: 2010-10-01 Cal Date: 2010-10-01 > >INFO Z2234567 SWITCH … OK > |
| 2 | BAND x BAND | Sets baud rate to x bits per second. Allowed values are 9 600, 19 200, 38 400, 57 600, and 115 200. If no argument is provided, the current rate is displayed. Note that the baud rate change takes effect after the prompt in response to the command is sent/ 0~31 Bytes: Description After 31 Bytes: Items | >BAND Baud: 9600 >baud 115200 > |
| 3 | ECHO ON ECHO OFF ECHO | Sets command line echoing to “ON” or “OFF”. If no argument is provided, the current setting is displayed 0~31 Bytes: Description After 31 Bytes: Items | >ECHO ECHO: ON >ECHO OFF OK > >ECHO ECHO: OFF > |
| 4 | SW X Y M N SW X Y SW X SW | Set switch x to M->N state X: Valid switch numbers range from 1 to the number of installed switches. Y: Y is as below values POS: Switch actual position SPOS: Switch setting position CNT: Set switch count (Valid after passwd command) M: Input Channel: From 1 to switch channels N: Output Channel: From 1 to switch channels Displays status of switch x y parameter. Y: Y is as below values POS: Switch actual position SPOS: Switch setting position CNT: Switch Life (Increase 1 after switching) 0~31 Bytes: Description After 31 Bytes: Items Displays status of Switch x. Displays status of all switches installed. Note: If the switch is not online, response Err: Not Online in read and write command. | Set switch 2 to 2->4 state >SW 2 SPOS 2 4 OK > Get switch 3 actual State >SW 3 POS SW 3 POS: 1->8 > Get switch 2 status >SW 2 SW 2 POS: 2->4 SW 2 SPOS: 2->4 SW 2 CNT: 1000 Get all switch all State >SW SW 1 POS: 1->2 SW 1 SPOS:1->2 SW1 CNT: SW 2 POS: 2->6 SW 2 SPOS: 2->6 SW 2 CNT …. > |
| 5 | MSW X | Set all switches state X: AA,BB,CC;AA,BB,CC;AA,BB,CC; AA: Switch Index (From 0 to switch installed; If AA is 0, set all switch to the same state. Ignore all switches are not installed or failed.) BB: Input Channel (Switch Input Channel) CC: Output Channel (Switch Output Channel) Note: Ignore all switches not installed. | Set switch 1 to 1-> 2State and Switch 2 to 1->5 State. >MSW 1, 1, 2; 2, 1, 5;……… OK > Set all switches to 1->3 State >MSW 0, 1, 3; OK |
| 6 | PASSWD | Let the switch’s life and manufacture information command can be set. | >PASSWD SUPERUSER OK > |
| 7 | RST | Soft reset the module. | >RST OK > |
| 8 | FWDL | 1K/128 XModem Refer to XModem protocol for detailed procedure/protocols of firmware upgrade | >FWDL CCC… |
| 9 | LOCAL | Set local to “ON” or “OFF”. If no argument is provided, the current setting is displayed 0~31 Bytes: Description After 31 Bytes: Items | >Local Local: On > Set to remote state >local off >ok |
| 10 | Alarm X Alarm | Displays status of alarm x parameter. X: X is as below values SW: Switch fail alarm Displays all alarms 0~31 Bytes: Description After 31 Bytes: Items | >Alarm SW SW1 Alarm: On SW3 Alarm: On > >Alarm SW1 Alarm: On SW3 Alarm: On > |

    - Firmware Upgrade procedures

The Switch module shall be able to upgrade its FW via the serial communication. It must keep the normal operation during the FW download process. The module shall be able to use the previous FW version if the upgrade process fails (i.e. the Switch module shall have two areas in the non-volatile memory to download the FW code without discarding the currently executing code).

The FW download will be carried out as follows.

The host sends the “FWDL” command. Switch module response ‘C’ to start XModerm protocol.

Please refer to XModem protocol for detailed procedure/protocols to transfer the firmware image file.

After firmware upgrading, the switch module will reset itself and load new firmware version.